WIN NT Parallel Port Setup:

The files included in \windows\ntdriver:

-------------------------------------------------------------------------------

Promice.bat	- A batch file to install the PromICE parallel driver

Promice.ini	- The parallel port driver settings file

Promice.sys	- The PromICE parallel driver system file

Regini.exe 	- The Microsoft REGINI program

Step by Step Instructions:

------------------------------------------------------------------------------

In order to use the parallel port for fast downloading, follow these

steps to install the parallel port driver:

1. Open an MS-DOS Prompt window, and use the following to install

LoadICE parallel driver installation files on your system.  For

example:

cd c:\loadice

md driver

copy a:\windows\ntdriver\*.*  c:\loadice\driver

2. Reboot your computer and access your computer's BIOS setup screens.

3. While in BIOS setup, be sure that the parallel port is in either

Standard or EPP mode. Do not use bi-directional, ECP or DMA modes.

4. While in BIOS setup, note the address of the parallel port.  This

is very important.  Some new computers may not use the standard

addresses.

5. Save any changes and exit BIOS.  Reboot and login to Windows as

Administrator.

6. Edit the PromICE.ini file in the directory where you installed the

ntdriver files, c:\loadice\driver in the example.  Change the

IoPortAddress to match the address of your parallel port.

7. If necessary, edit the PromICE.bat file.  This should only be

necessary if you installed Windows NT system into a directory other

than C:\winnt.

8. Run PromICE.bat.  This bat file copies Promice.sys  into your

computer's WinNT system directory.  Then it uses Regini.exe to record

the settings in Promice.ini.

9. Restart the computer.

10. From the Start Menu, select Settings->Control Panel.

11. In Devices Control Panel, locate the service called "PromICE".  It

should be started and automatic.

12. In case of problems, recheck the BIOS and PromICE.ini settings

then rerun PromICE.bat and reboot.

If you can't connect to the PromICE via the parallel port, check your

settings and retry.

See the PromICE User Manual for more information.